| GPIO | Input | Output | Notes | Used as | Type | Pin |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | pulled up | OK | outputs PWM signal at boot |  |  |  |
| 1 | TX pin | OK | debug output at boot |  |  |  |
| 2 | OK | OK | connected to on-board LED | IR | RMT | 2 |
| 3 | OK | RX pin | HIGH at boot |  |  |  |
| 4 | OK | OK |  | RFID IRQ |  | 4 |
| 5 | OK | OK | outputs PWM signal at boot | Trigger | GPIO | 5 |
| 6 | x | x | connected to the integrated SPI flash |  |  |  |
| 7 | x | x | connected to the integrated SPI flash |  |  |  |
| 8 | x | x | connected to the integrated SPI flash |  |  |  |
| 9 | x | x | connected to the integrated SPI flash |  |  |  |
| 10 | x | x | connected to the integrated SPI flash |  |  |  |
| 11 | x | x | connected to the integrated SPI flash |  |  |  |
| 12 | OK | OK | boot fail if pulled high | Laser | GPIO | 12 |
| 13 | OK | OK |  | RGB Pixels 1 (Gun) | RMT | 13 |
| 14 | OK | OK | outputs PWM signal at boot | PowerUp-Button | GPIO | 14 |
| 15 | OK | OK | outputs PWM signal at boot | RFID Reader SPI | Reset | 15 |
| 16 | OK | OK |  | RGB Pixels 2 (Gun) | RMT | RX2 |
| 17 | OK | OK |  | RGB Pixels (Vest) | RMT | TX2 |
| 18 | OK | OK |  | Capacitive Touch Display | I2C | 18 |
| 19 | OK | OK |  | Capacitive Touch Display | I2C | 19 |
| 21 | OK | OK |  | SD Card RFID Reader SPI | SPI1_MISO | 21 |
| 22 | OK | OK |  | SD Card RFID Reader SPI | SPI1_MOSI | 22 |
| 23 | OK | OK |  | SD Card RFID Reader SPI | SPI1_CLOCK | 23 |
| 25 | OK | OK |  | SD Card | ChipSelect | 25 |
| 26 | OK | OK |  | Speaker | I2S_BCK | 26 |
| 27 | OK | OK |  | Speaker | I2S_Data | 27 |
| 32 | OK | OK |  | Speaker | I2S_WS | 32 |
| 33 | OK | OK |  | RFID Reader SPI | ChipSelect | 33 |
| 34 | OK |  | input only | IR Receiver 1 (Gun) |  |  |
| 35 | OK |  | input only | IR Receiver 2 (Gun) |  |  |
| 36 | OK |  | input only | IR Receiver 1 (Front) |  |  |
| 39 | OK |  | input only | IR Receiver 2 (Back) |  |  |

|  |  | SD Card - SPI1 CLOCK |
| --- | --- | --- |
| IR Receiver 1 – Front (RMT) |  | SD Card - SPI1 MOSI |
| IR Receiver 2 – Back (RMT) |  | RGB Pixels 2 – Gun (RMT) |
| IR Receiver 1 – Gun (RMT) |  |  |
| IR Receiver 2 – Gun (RMT) |  | SD Card - SPI1 MISO |
| Speaker - I2S_WS |  | Capacitive Touch (I2C) |
| RFID - Chip Select |  | Capacitive Touch (I2C) |
| SD Card – Chip Select |  | Trigger (GPIO) |
| Speaker - I2S_BCK |  |  |
| Speaker – I2S_Data |  |  |
| PowerUp-Button (GPIO) |  | RFID IRQ |
| Laser (GPIO) |  | IR (RMT) |
| RGB Pixels 1 – Gun (RMT) |  | RFID Reset |
|  |  | RGB Pixels 1 – Vest (RMT) |

| 3.3V |  | GND |
| --- | --- | --- |
|  |  | SD Card - SPI1 CLOCK |
| IR Receiver 1 – Front (RMT) |  | SD Card - SPI1 MOSI |
| IR Receiver 2 – Back (RMT) |  | RGB Pixels 2 – Gun (RMT) |
| IR Receiver 1 – Gun (RMT) |  |  |
| IR Receiver 2 – Gun (RMT) |  | SD Card - SPI1 MISO |
| Speaker - I2S_WS (LRC) |  |  |
| RFID - Chip Select (SDA) |  | Capacitive Touch (I2C) |
| SD Card – Chip Select |  | Capacitive Touch (I2C) |
| Speaker - I2S_BCK |  | Trigger (GPIO) |
| Speaker – I2S_Data |  | PSRAM |
| PowerUp-Button (GPIO) |  | PSRAM |
| Laser (GPIO) |  | RFID IRQ |
|  |  | RGB Pixels 1 – Vest (RMT) |
| RGB Pixels 1 – Gun (RMT) |  | IR (RMT) |
| SPI FLASH |  | RFID Reset |
| SPI FLASH |  | SPI FLASH |
| SPI FLASH |  | SPI FLASH |
|  |  | SPI FLASH |

- LRC (Left/Right Clock) - this is the pin that tells the amplifier when the data is for the left channel and when its for the right channel

- BCLK (Bit Clock) - This is the pin that tells the amplifier when to read data on the data pin.

- DIN (Data In) - This is the pin that has the actual data coming in, both left and right data are sent on this pin, the LRC pin indicates when left or right is being transmitted

## Gain

GAIN is, well, the gain setting. You can have a gain of 3dB, 6dB, 9dB, 12dB or 15dB.

- 15dB if a 100K resistor is connected between GAIN and GND

- 12dB if GAIN is connected directly to GND

- 9dB if GAIN is not connected to anything (this is the default)

- 6dB if GAIN is connected directly to Vin

- 3dB if a 100K resistor is connected between GAIN and Vin

This way, the default gain is 9dB but you can easily change it by tweaking the connection to the GAIN pin. Note you may need to perform a power reset to adjust the gain.

# SD / MODE

This pin is used for shutdown mode but is also used for setting which channel is output. It's a little confusing but essentially:

- If SD is connected to ground directly (voltage is under 0.16V) then the amp is shut down

- If the voltage on SD is between 0.16V and 0.77V then the output is (Left + Right)/2, that is the stereo average.

- If the voltage on SD is between 0.77V and 1.4V then the output is just the Right channel

- If the voltage on SD is higher than 1.4V then the output is the Left channel.

This is compounded by an internal 100K pulldown resistor on SD so you need to use a pullup resistor on SD to balance out the 100K internal pulldown.